# CIDR Notation & IP Classes (Networking Notes)

## What is CIDR?

CIDR (Classless Inter-Domain Routing) is a method used to allocate IP addresses efficiently and divide networks into smaller subnets.

Instead of using only fixed classes (A, B, C), CIDR allows flexible network sizes.

Syntax:

IP_Address/Prefix_Length

### Example

192.168.1.0/24

- 192.168.1.0 → Network Address

- /24 → First 24 bits are Network bits

- Remaining 8 bits are Host bits

# CIDR Formula

Network Bits = Prefix Length

Host Bits = 32 − Prefix Length

### Total IP Addresses

2^(Host Bits)

### Usable IP Addresses

2^(Host Bits) - 2

Note: Two IP addresses are reserved.

- Network Address

- Broadcast Address

# CIDR Cheat Sheet

| CIDR | Subnet Mask | Total IPs | Usable IPs |
| --- | --- | --- | --- |
| /32 | 255.255.255.255 | 1 | 1 |
| /30 | 255.255.255.252 | 4 | 2 |
| /29 | 255.255.255.248 | 8 | 6 |
| /28 | 255.255.255.240 | 16 | 14 |
| /27 | 255.255.255.224 | 32 | 30 |
| /26 | 255.255.255.192 | 64 | 62 |
| /25 | 255.255.255.128 | 128 | 126 |
| /24 | 255.255.255.0 | 256 | 254 |
| /23 | 255.255.254.0 | 512 | 510 |
| /22 | 255.255.252.0 | 1024 | 1022 |
| /16 | 255.255.0.0 | 65,536 | 65,534 |
| /8 | 255.0.0.0 | 16,777,216 | 16,777,214 |

# IPv4 Address Classes

IPv4 addresses are divided into five classes.

| Class | First Octet | Default Mask | CIDR | Purpose |
| --- | --- | --- | --- | --- |
| A | 1 – 126 | 255.0.0.0 | /8 | Large Networks |
| B | 128 – 191 | 255.255.0.0 | /16 | Medium Networks |
| C | 192 – 223 | 255.255.255.0 | /24 | Small Networks |
| D | 224 – 239 | N/A | N/A | Multicast |
| E | 240 – 255 | N/A | N/A | Reserved |

# Class A

### Range

1.0.0.0 - 126.255.255.255

### Default Mask

255.0.0.0

### CIDR

/8

### Structure

Network.Host.Host.Host

### Example

10.0.0.0/8

# Class B

### Range

128.0.0.0 - 191.255.255.255

### Default Mask

255.255.0.0

### CIDR

/16

### Structure

Network.Network.Host.Host

### Example

172.16.0.0/16

# Class C

### Range

192.0.0.0 - 223.255.255.255

### Default Mask

255.255.255.0

### CIDR

/24

### Structure

Network.Network.Network.Host

### Example

192.168.1.0/24

# Class D

### Range

224.0.0.0 - 239.255.255.255

### Purpose

- Multicast Communication

- Not assigned to hosts

# Class E

### Range

240.0.0.0 - 255.255.255.255

### Purpose

- Reserved for research and testing

# Private IP Address Ranges

| Class | Private Range |
| --- | --- |
| A | 10.0.0.0 – 10.255.255.255 |
| B | 172.16.0.0 – 172.31.255.255 |
| C | 192.168.0.0 – 192.168.255.255 |

# Public vs Private IP

| Public IP | Private IP |
| --- | --- |
| Internet accessible | Used inside local network |
| Globally unique | Can be reused in different networks |
| Assigned by ISP | Assigned by organization/router |

# CIDR Examples

### Example 1

192.168.1.0/24

- Network Bits = 24

- Host Bits = 8

- Total IPs = 256

- Usable IPs = 254

### Example 2

10.0.0.0/16

- Network Bits = 16

- Host Bits = 16

- Total IPs = 65,536

- Usable IPs = 65,534

### Example 3

172.16.1.0/30

- Total IPs = 4

- Usable IPs = 2

Used for Point-to-Point links.

# Azure Networking Example

### Virtual Network

10.0.0.0/16

Inside the VNet:

10.0.1.0/24
10.0.2.0/24
10.0.3.0/24

VNet (10.0.0.0/16)
│
├── Subnet-1 (10.0.1.0/24)
├── Subnet-2 (10.0.2.0/24)
└── Subnet-3 (10.0.3.0/24)

This concept is used in Azure, AWS, and Google Cloud.

# Memory Trick

Class A → /8
Class B → /16
Class C → /24

For Subnetting:

/24 → 256 IPs
/25 → 128 IPs
/26 → 64 IPs
/27 → 32 IPs
/28 → 16 IPs
/29 → 8 IPs
/30 → 4 IPs

# Interview Questions

### Q1. What is CIDR?

CIDR is a method of allocating IP addresses and creating subnets using a prefix length (for example, /24) instead of fixed IP classes.

### Q2. How many usable IPs are available in a /24 network?

- Total IPs = 256

- Usable IPs = 254

### Q3. What are the default CIDR values for Class A, B, and C?

| Class | Default CIDR |
| --- | --- |
| A | /8 |
| B | /16 |
| C | /24 |

### Q4. Which private IP ranges are commonly used?

- 10.0.0.0/8

- 172.16.0.0 – 172.31.255.255

- 192.168.0.0/16

# Summary

- CIDR provides flexible IP allocation.

- CIDR uses /Prefix Length.

- IPv4 has five classes: A, B, C, D, and E.

- Private IPs are used inside internal networks.

- Azure, AWS, and GCP use CIDR for VPC/VNet and subnet creation.